Experiment Title: Enzyme Kinetics Assay for Lactate Dehydrogenase (LDH) Activity

Date: February 22, 2025
Researcher: Len
Lab/Project: Metabolic Enzyme Characterization
Objective:
To quantify the enzymatic activity of lactate dehydrogenase (LDH) using a colorimetric NADH oxidation assay.

Materials & Reagents:

- Reagents:

  - NADH (10 mM stock)

  - Pyruvate (10 mM stock)

  - Tris-HCl buffer (50 mM, pH 7.4)

  - LDH enzyme (purified, 1 mg/mL)

  - Deionized water

- Equipment:

  - UV-Vis spectrophotometer (340 nm)

  - 96-well plate

  - Multichannel pipette

  - Plate reader

Procedure:

- Prepare assay buffer: Mix Tris-HCl buffer with NADH to obtain a final concentration of 0.2 mM NADH.

- Prepare reaction mix: Add pyruvate to obtain a final concentration of 1 mM.

- Dispense 200 µL of reaction mix into each well of a 96-well plate.

- Add 5 µL of LDH enzyme (0.5 µg/well) to start the reaction.

- Immediately measure absorbance at 340 nm every 30 seconds for 5 minutes.

- Perform blank correction using wells containing buffer only.

- Calculate enzyme activity based on the decrease in NADH absorbance (ε = 6220 M⁻¹cm⁻¹).

Results & Observations:

- NADH absorbance at 340 nm decreased over time, confirming enzyme activity.

- Initial velocity (V₀) calculated using linear regression: 0.18 ± 0.02 ΔA340/min.

- Enzyme activity: 12.4 U/mL (calculated based on NADH oxidation rate).

Conclusion & Next Steps:

- The assay successfully quantified LDH activity with reproducible results.

- Next, optimize the pyruvate concentration to determine enzyme kinetics (Km, Vmax).

- Validate results with a control enzyme sample.