Experiment Title: Optimization of Buffer Conditions for LDH Assay

Date: February 7, 2025
Researcher: Len
Lab/Project: Enzyme Assay Development
Objective:
To determine the optimal buffer composition for maintaining lactate dehydrogenase (LDH) activity and stability in the enzymatic assay.

Materials & Reagents:

- Buffers to be tested:

  - Tris-HCl (50 mM, pH 7.4, 8.0, 8.5)

  - HEPES (50 mM, pH 7.4, 8.0, 8.5)

  - Phosphate buffer (50 mM, pH 7.0, 7.4, 7.8)

- Reagents:

  - NADH (10 mM stock)

  - Pyruvate (10 mM stock)

  - LDH enzyme (1 mg/mL)

  - Deionized water

- Equipment:

  - UV-Vis spectrophotometer (340 nm)

  - 96-well plate

  - pH meter

  - Multichannel pipette

Procedure:

- Prepare buffer solutions at specified pH values and confirm using a calibrated pH meter.

- Prepare reaction mixtures with each buffer and a final NADH concentration of 0.2 mM.

- Add pyruvate (1 mM) to initiate the reaction.

- Dispense 200 µL of reaction mix into each well of a 96-well plate.

- Add 5 µL of LDH enzyme to each well and immediately measure absorbance at 340 nm every 30 seconds for 5 minutes.

- Perform blank correction using wells containing buffer only.

- Compare enzyme activity in different buffers by analyzing the rate of NADH oxidation.

Results & Observations:

- LDH activity was highest in Tris-HCl, pH 7.4, with an initial velocity of 0.22 ± 0.01 ΔA340/min.

- HEPES buffer showed moderate activity but a slower reaction rate at pH 8.5.

- Phosphate buffer at pH 7.0 resulted in reduced activity, suggesting possible enzyme instability.

Conclusion & Next Steps:

- Tris-HCl (pH 7.4) is the most suitable buffer for maintaining LDH activity.

- Further tests will explore ionic strength and cofactor stabilization.

- Investigate buffer effects on enzyme storage stability over time.